Release Notes

| Program: AVF216L | Revision Number: 6.20.1129’17 Replaced Revision: 6.20.0601’17 Creation Date: 11/29/2017 |  |
| --- | --- | --- |
| Author or Designer: Shin |  |  |
| What Changed? | SavingTime=0, EnabledSaving=0, EnabledPowerOff=0, PowerOffTime=0 |  |
| What has been Fixed? | Avscan_n.dll - Set the window style to TOPMOST for multifeed asking dialog. - Multifeed detected ask message changed. - Support asking user for multifeed reaction. - Support edge cleanup for ID card mode. - Convert the lineart to gray image to transfer for some Apps cannot recognize lineart image. - Bug fixed to allocate wrong memory size. - Changes for auto level algorithm. - Support image processing of selective color. - Bug fixed no response problem in checking multifeed image when blank page removal is enable. - Modify for warming up scanner. - Fixed the crashed problem when in insufficient memory. - Changes the clean up size from 2mm to 2.5mm. - Bug fixed to send wrong background color for HW cropping. - Changes the clean up size from 3mm to 2mm. - Add the host name into job information. - Changes for sending scanner job information. AVF216L.ds - Support to read/send input tray position. - New custom capability CAP_TRAYPOSITION. - Extend the size of structure CUSTOMDATAEX to 696 bytes, appeend 129 bytes for reserved. - Changes for individual ultrasonic control. - Support ultrasonic sensor and multifeed reaction parameters for DAT_CUSTOMDSDATA. - New custom capability CAP_MULTIFEEDREACTION. - Support the reaction of multifeed happened to stop and show error immediately or ask user if keep the image. - New custom capability ICAP_LONGPAGEHEIGHT for long paper scanning in non-UI. - Bug fixes for multifeed error code and message are not stored. - Support control individual ultrasonic sensor. - Define the minimum and maximum scan resolution in INI. - Support staple detection. - Fixed scan resolution issue. - Changes for custom capability CAP_SCANNEROPERATION. - Fixed capability ICAP_AUTOMATICBORDERDETECTION sending causes to disable long paper cropping. - Changes for Inspector Twain with named "InspectorTwainCore". - Fixed flated scan is disabled. - Export the imprinter model name and firmware version. - Modify for NTS. - Modify codes for MSG_STOPFEEDER. - Do not reset the minimum area with imprinter installed. - Fixed the wrong settings when power saving is disabled. Twdsm_n.dll - Changes for input try position control. - Add functions AVResetCalibrationData and AVDoCalibrationData for scanner calibration. - Bug fixes that the progress bar cannot be closed if DestroyWindow is failed. - Add new description of generation operation. - Get the physical ethernet MAC and IP address of local computer. Advance.dll - Support Tray Position setting. - Modify Feeding Group Box size, the group box will change size according to the components quantity. - Claibration button support Newavapi. - Change multifeed sensor select style on ui. - Modify multifeed reaction combo box stypl to drop list. - Support asking user for multifeed reaction. - IRIS product change string: From Automatic change to Deskewing. - Add :(colon) after UltraSonic string when UltraSonic sensor selection display. - Support ultrasonic sensor. - Support StapleDetection. - Fix after preview scan UI become to background problem. - Fix preview B&W memory does not release problem. - Fix when scan source is flatbed select paper size incorrect problem(UDS document scanner). - Fix the Xoffset error when previewing(AD7080 select custom scan area with 1x1 inch after flatbed preview,PCM BUG-2017000369) - Fix when cropping selected 4: Automatic multiple Yoffset doesn't disable problem. - Fix sometimes Power On Time incorrect(more than a minute). - Fix multi-feed PaperLength incorrect when user select a custom scan area.(PLM BUG-2017000865) - Fix flatbed scanner after preview UI crash problem. - Max value of Noise Number from 3600 change to 255 in TabEnhancement. - When NewAvapi, imprinter only show model name and firmware version in TabInformation . - The maximum value of sensitivity for auto color detection change to 99. - Modify Preview scan area incorrect problem(PLM BUG-2017000369). |  |
| Known Defects | 稅務專案 |  |
| Required Files And Revision Level | File Name | Revision |
|  | Avscan_n.dll | 4.0.0.28 |
|  | AVF216L.ds | 4.0.0.36 |
|  | Twdsm_n.dll | 4.0.0.11 |
|  | Advance.dll | 4.0.0.30 |
|  | AVF216L.dll | 0.9.2017.502 |
|  | SmartImage.dll | 1.1.201.0 |
|  | AVIP.dll | 1.0.0.1 |
|  | AVJpegRAWToJFIFLib.dll | 1.5.16.0 |
|  | Cmatch.dll | 4.3.9.4 |
|  | AVBarcodeEx.dll | 1.0.0.7 |
|  | Av2PDF.dll | 1.0.0.8 |
|  | fpcreator_img.dll | 1.0.0.617 |
| ISIS Driver |  |  |
| Details: |  |  |