Ultra Electronics Card Systems

Waverley House

Hampshire Road

Granby Estate

Weymouth

Dorset

DT4 9XD

United Kingdom

SMART ENCODING DESCRIPTION

FOR THE MAGICARD

PRONTO,

ENDURO,

RIO PRO,

PRINTER FAMILY

 ULTRA ELECTRONICS LIMITED, 2011

This document contains confidential and proprietary information belonging to Ultra Electronics Limited and may not wholly or partially be copied, stored in a data retrieval system, disclosed to third parties or used for any purpose other than that for which it was supplied without the express written authority of Ultra Electronics Limited, Card Systems.

When no longer required for the purposes authorised or any time upon request by Ultra Electronics Limited, this document shall be returned to Ultra Electronics Limited, Card Systems.

Any enquiries about this document or its contents should be made in the first instance to:

Card Systems Customer Support Manager
		Ultra Electronics Card Systems
		Waverley House
		Hampshire Road
		Granby Estate
		Weymouth
		Dorset
		DT4 9XD

United Kingdom

Tel:	(01305) 767100

Fax:	(01305) 777904

Amendment Control

Amendments to this document will be made on agreement with the designated authorisers.

Amendment Record

| Issue/ Amend No | Section/ Page | Change Details | Incorporated By Date |  |
| --- | --- | --- | --- | --- |
| 1.00 |  | Initial Release | JH | 27/01/2011 |
| 1.01 | 2./p 4 | Added MagAPI.dll file location data | JH | 16/02/2011 |
| 1.02 | 4./p7 | Updated links to latest API docs. Added Appendix E (example application links) | JH | 24/06/2011 |
| 1.03 | 4./p7 | Updated Appendix B – changed the download from just API documentation to all API files (including dll, def, lib and hdr files) | JH | 11/01/2012 |

Contents

1.   Purpose of the API’s…………………………………………………………………………	pg 4

2.   API File………………………………………………………………………………………..	pg 4

3.   Smart Encoding process…..………………………………………………………………..	pg 4

4.   Appendixes:-

Appendix A – Process flow chart…….……………………………………………………	pg 6

Appendix B – Magicard API documents download ……….………………………………	pg 7

Appendix C – Omnikey 5321 Encoder downloads …….…………………………………	pg 7

Appendix D – Magicard printer driver downloads ………………………………………	pg 7

Appendix E – Example application download …..………………………………………	pg 7

# 1.  Purpose of the API’s

1.1   The purpose of the Magicard API is to allow applications to synchronize their operations with the operation of the printer.

1.2   It provides functions to enable placement of cards in the correct position for contact and contactless chip encoding and to eject cards from the printer.

  - It also provides feedback to the application to confirm completion of the above operations.

- It is important to note that the API’s cannot be used to send encoding or print data to the printer – the API’s are intended to be used only to position the card within the smart chip encoder.

# 2.  API File

2.1   The API itself can be used by loading the MagAPI.dll file.  A ‘C’ header file and a .def file are provided.

2.2   The MagAPI.dll file is loaded automatically during printer driver installation to the following locations :-

- XP & Vista (32 bit)	-	C:\Windows\System32

- XP & Vista (64 bit)	-	C:\Windows\SysWOW64

- Windows 7 (32 bit)	-	C:\Windows\System32

- Windows 7 (64 bit)	-	C:\Windows\SysWOW64

# 3.  Smart Encoding process

3.1   For each encoding/print job, the following  5 step process should be performed :-

  - Note: also see the flow chart in Appendix A below

STEP 1 :  The 3rd party software should use the Magicard APIs to control the card positioning into the encoder.

- Note: see Appendix B below for details of the Magicard APIs.



STEP 2 :   The 3rd party software should then communicate with the encoder using the encoder’s own driver files. All reading and writing to the card should be carried out at this stage.

Note: The factory fitted smart encoder in the Pronto, Enduro and Rio Pro printers is an Omnikey 5321 encoder - see Appendix C below for details of the 5321 driver files.



STEP 3 :   Once encoding is complete, the 3rd party software should send the image to be printed - via the Magicard printer driver and Windows spooler.

- Note: see Appendix D below for details of the Magicard driver files.



STEP 4 :   When the printer receives the image data from the spooler it will automatically:-

- move the card from the encoding position to the printing position

- print the image onto the card

when printing is complete, the card will be ejected from the printer



STEP 5 :   The 3rd party software should then use the API to position the next card into the encoder, and so on.

Appendix A:

Process flow chart:

<<< All reading  and writing to the smart card should be carried out at this stage.

The 3rd party software uses the API’s to command the printer to position a card in the encoder

The printer takes a card from the feed hopper and moves it into the encoder

The printer returns

success/failure  indication to the software via the API

The 3rd party software communicates with the encoder using the encoder’s own driver files.

The 3rd party software sends the image to be printed - via the Magicard printer driver and Windows spooler.

The printer receives the image data and:-

moves card to printing position

prints the image onto the card

Ejects the card

The printer returns

success/failure  indication to the software via the API

START

Appendix B:

Magicard API files download:

The latest Magicard API files - including dll, definition, header and library files, and a developer’s guide (Magicard API.pdf), can be found in :-

- the UltraDriver CD shipped with every printer

(in the D:\ AutoPlay\Docs\Developer Information folder)

- our website at:

http://www.ultramagicard.com/fileadmin/inc/support_scripts_en/printers/files/magicard_api_files.zip

Appendix C:

Omnikey 5321 Encoder downloads:

All information on this encoder can be found on HID’s website at:

http://www.hidglobal.com/prod_detail.php?prod_id=171

The most up-to-date drivers for this encoder can be downloaded from:

http://www.hidglobal.com/driverDownloads.php?techCat=19&prod_id=171

Appendix D:

Magicard printer drivers downloads:

The most up-to-date drivers for the Magicard printers can be downloaded from our website at::-

- Pronto:  http://www.ultramagicard.com/technical-support/pronto/drivers/

- Enduro:  http://www.ultramagicard.com/technical-support/enduro/drivers/

- Rio Pro:  http://www.ultramagicard.com/technical-support/rio-pro/drivers/

  - Note: A printer must be connected to the PC USB port in order to complete the USB driver installation.

Appendix E:

Example application download:

An example application (with source code) can be downloaded from our website at::-

http://www.ultramagicard.com/fileadmin/inc/support_scripts_en/printers/files/api_example_app_&_source.zip