TCP Demo (ver 2.30) user’s manual

Applicable OS : Windows 98, Me, NT, 2000, Xp（RS-232C）

Windows 98SE, Me, 2000, Xp（USB）

Purpose : For sales demonstration, exhibition, making card samples.

Program files : This application was made with DLL file. (You don’t need to install OCX driver.) Since this DLL file is no more supported by Star, source code cannot be disclosed in order to avoid the future trouble. In case of the reference of application developing, please refer OCX sample.

Installation : Click Setup.exe in the folder of “TCP Demo”

After installation, shortcut will be made in program menu in the windows menu.

How to Use (Preparation)

- Execute TCPDemo.exe in the program menu of windows menu.

- Please set the communication condition. If you need to change the setting, please click the condition and change the BOX to be inverted character like below Port.

Example, if you need to change the Port from COM1 to COM4, click the box and change the color of COM4 to inverted one.

- When you click the “Set”, then communication starts. If it success, firmware version is shown at the bottom of left. If below message remains, communication is not success. You need to check the setting or connecting again.

- In order to TCP ZU Series (USB I/F), you have to install USB vender class driver at the first connection.  After install vender class driver, please check com port number for USB connection by using DeviceView.exe as followings.

- After initialization of TCP, you can see below main menu.

6. To confirm the correct communication, click “L”(Initialize). And please confirm message window like below popped up and disappeared, and TCP is executed the reset.

How to Use (Each Button)

A (User command mode)

When you click this button, below window is coming up.

B (message box)

You can input text data for printing. In order to “Line Feed”, you can use RETURN key.

C,D,E (text data parameter)

You can select the text data direction for executing C.

“2” = Vertical

“3” = Horizontal

D, E  (text data position)

You can fix the position (X coordinate and Y coordinate) of text box B.

TCP300	Series:	In case of F = 2, D can be from 0 to 319, and E can be from 0 to 479.

In case of F = 3, D can be from 0 to 479, and E can be from 0 to 319.

TCP400	Series:	In case of F = 2, D can be from 0 to 319, and E can be from 0 to 551.

In case of F = 3, D can be from 0 to 551, and E can be from 0 to 319.

This number is for executing J.

J (text data sending)

When you click, text data B are sent to TCP buffer with parameter C,Dand E.  It is the same function of the command 41h.

F,G, H, I,(image data parameter)

You can select the text data direction for executing F.

“2” = Vertical

“3” = Horizontal

G (image data number)

You can select the image number which already stored into flash ROM of TCP. This number is available for executing “Y”. You can put the number from 0 to 1999.

H, I  (image data position)

You can fix the position (X coordinate and Y coordinate) of image NoG. The parameter is for executing “X”.

TCP300	Series:	In case of F = 2, H can be from 0 to 319, and I can be from 0 to 479.

In case of F = 3, H can be from 0 to 479, and I can be from 0 to 319.

TCP400	Series:	In case of F = 2, H can be from 0 to 319, and I can be from 0 to 551.

In case of F = 3, H can be from 0 to 551, and I can be from 0 to 319.

This number is for executing K.

L (Reset button)

When you click, reset is executed .  It is the same function of the command 5Fh.

M (clear buffer button)

When you click, printing data of text and internal image in the buffer of TCP are cleared. It is the same function of the command 40h.

N (erase button)

When you click, printing area erasing is executed.  It is the same function of the command 48h.

O (over print button)

When you click, over print (without erasing) is executed.  It is the same function of the command 47h.

P (erase and print button)

When you click, erasing and printing are executed.  It is the same function of the command 46h.

Q (eject card button)

When you click, card is ejected from TCP.  It is the same function of the command 50h.

R (response data)

When you communicate with TCP, the response (status) code is shown at here.

S (Save Text button)

You can save the text data “E” to the file of “Tcp300.ini”. It should be located in the same folder.

User command mode

A-1 (User Command) --- You can input arbitrary command. For example, in case of RESET command sending, you just input “5F” to this text box.

A-2 (Data parameter box) --- You can input arbitrary parameter if the command requires the parameter. For example, in case of [printing data setting], input “41” to A-1, and input like “2,0,23,PRINT DATA STRING” to this text box.

A-3 (executing button) --- When you click this button, command and parameter are sent to TCP.

A-4 (Reset button) --- When you click this button, TCP executes RESET.

A-5 (response data box) --- It shows the text data of response from TCP after command execution.

A-6 (Check box for Magnetic Write) --- Check here for magnetic data R/W since TCP only Magnetic head for each track. You can select the magnetic format of ISO or JIS in A-7.

A-9 (Sending data button for each Magnetic Track) --- When you click this button, magnetic strip data in A-8 is sent to TCP.

A-10 (Excute Magnetic Read button) --- Read selected magnetic stripe data

A-11 (Excute Magnetic Write button) ---Write data to selected magstripe

A-12 (response data box) --- It shows the text data of response from TCP after command execution.